Technical Specification for U-Frame Board Loader/Unloader

U型架投收板机设备技术规格书

- Equipment Overview设备概述

- Equipment Name设备名称：U-Frame Board Loader/Unloader U型架投收板机

- Application用途：（1）Automatic Board Loading and Unloading自动投板、收板；（2）Intelligent Functions智能化功能

- Process Flow工艺流程

Loader:

Feeding Process: Trolley Transportation → Board Gripping & Retrieval from U-Frame→ Transfer to Conveyor Line → Positioning → Conveyed to Main Line

投板机：

入料：台车运载→从U型架抓料取板→转移至传输线→定位→输送至主线。

Unloader:

Board Discharging from Main Line → Sensor-Assisted Board Positioning → Board Gripping → Transfer to U-Frame→ Discharging: Trolley Transportation

收板机：

主线出板→传感器辅助定位板件→抓取板件→转移至U型架→出料：台车运载。

- Overall equipment description设备整机说明

- The whole machine must be brand new equipment, and the accessories delivered with the equipment must be brand new products整机是全新设备，随机附件必须是全新配件。

- Modular design, the machine consists of different functional modules模块化设计，机器由不同的功能模块组成。

- It must be ensured that the whole equipment can be used normally under the power supply of three-phase five wire 380V ± 10%, 50HZ必须保证整机能在三相五线380V±10%、50HZ电源下正常使用。

- The appearance of the whole equipment is aesthetic, and the manufacturing materials and maintenance materials of the equipment meet environmental protection requirements整机外形美观，设备的制造材料和维护用品满足环保要求。

- The whole equipment has high reliability and low failure rate整机可靠性高, 故障率低。

- The equipment must be designed by the supplier / manufacturer and not involve any intellectual property disputes, otherwise any losses caused by it shall be borne by the supplier设备须自主设计，不涉及知识产权纠纷，否则由此造成的损失由供方承担。

- All HMI, operating computers (work software), labels, etc. require at least three languages: English, Chinese, and Thai所有人机界面、操作电脑（工作软件）、标签等至少需要英、中、泰三种语言。

- Delivery location: Equipment installation site交货地点：设备安装现场

- Process technical requirements (where items with * are key technical parameters, the same hereinafter)工艺技术要求（其中带*为关键技术参数，下同）

- General technical parameters通用技术参数

| No 序号 | Item 项目 | Technical requirements / specifications 技术要求/规格 | Supplier's Capability 供方能力 | Acceptance method 验收方法 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- |
| 1 | Maximum Panel Size 最大板件尺寸* | 24.5×28.5inch | 滿足需求 | Reviewed during 3D design review/On-site confirmation 3D会审时检查/现场检查 |  |
| 2 | Minimum Panel Size 最小板件尺寸* | 23.6×17.6 inch | 滿足需求 | Reviewed during 3D design review/On-site confirmation 3D会审时检查/现场检查 |  |
| 3 | Panel Thickness 板件厚度* | 0.3-3.2mm（include copper） 0.3~3.2mm(含铜) | 0.8-3.2mm垂直豎放翹曲小於10mm | On-site confirmation现场检查 |  |
| 4 | Panel Weight 板件重量* | 8 kg | 滿足需求 | On-site confirmation 现场检查 |  |
| 5 | Panel Surface Characteristics板件表面特性* | substrate, base copper, circuit, drilled holes, routed grooves, ultra-roughened surfaces, medium-roughened surfaces, Ink,etc 基材、基铜、线路、钻孔、锣槽、超粗化面、中粗化面、油墨等。 | 滿足需求 | On-site confirmation 现场检查 |  |
| 6 | Operating side location (direction) 操作面位置（方向） | Set according to the specific wiring 根据具体连线设置 | 以實際方案為準 | Reviewed during 3D design review/On-site confirmation 3D会审时检查/现场检查 |  |
| 7 | Incoming Direction 进板方向 | Pre-exposure loader and unloader: The 28-inch side of the board shall be parallel to the conveyor roller axis of the production line. 曝光前投收收板机：28inch的边与生产线的传送滚轮轴平行。 Others: The 24-inch side of the board shall be parallel to the conveyor roller axis of the production line. 其它：24inch的边与生产线的传送滚轮轴平行。 | 滿足需求 | Reviewed during 3D design review/On-site confirmation 3D会审时检查/现场检查 |  |
| 8 | Working height工作高度 | 900mm±30mm 1100mm±30mm | 以實際方案為準 | Reviewed during 3D design review/On-site confirmation 3D会审时检查/现场检查 |  |
| 9 | Production capacity 产能* | When board alignment positioning is activated, the production speed shall be ≥ 6 pieces/min. If the production speed requirement cannot be met for 8kg boards, the supplier shall specify the achievable production speed.​ For boards weighing 5kg or less, the production speed requirement shall be met. 拍板定位开启，产速≥6片/min。若生产8kg板件无法满足节拍要求时，供方写明可达到的产速。5kg以内板件需满足节拍要求。 | 0~5kg产速≥6片/min 5~8kg产速≥5.5片/min | Reviewed during 3D design review/On-site confirmation 3D会审时检查/现场检查 |  |
| 10 | Carrier 载具 | - U-Frame U型架。 - Be equipped with a barcode label on the side.侧面配备二维码标签。 - The screen printing loader shall be compatible with the integrated pallet trolley.丝印投板机需兼容一体式托盘台车。 | 以實際方案為準 | Reviewed during 3D design review/On-site confirmation 3D会审时检查/现场检查 |  |
| 11 | The material of the screws/nuts螺丝/螺帽材质 | 304 stainless steel 304不锈钢 | 滿足需求 | On-site confirmation 现场检查 |  |
| 12 | The material of the rack 机架材质* | 304 stainless steel 304不锈钢 | 滿足需求 | 现场检查 |  |
| 13 | Operation Interface 操作界面 | Use Touchscreen, Easy to Operate, Easy to Understand, and Intuitive Alerts 使用触摸屏，便于操作，容易理解，直观提醒 | 滿足需求 | On-site confirmation 现场检查 |  |
| 14 | Conveyor Bridge Support Design 输送过桥支撑设计 | For the independent conveyor wheels of the in-out board bridge support conveyor, each wheel shall be individually adjustable on the left and right sides perpendicular to the in-out board direction, so as to avoid conveyor interference with upstream and downstream equipment.进出板过桥支撑传送各独立传送轮需满足垂直于进出板方向左右侧可单独调整，避免与上下游设备传送干涉 | 滿足需求 | On-site confirmation 现场检查 |  |
| 15 | Workstation Design 工位设计 | - For board handling of the U-Frame, a traditional flipping board handling design shall be adopted. For board handling of the integrated pallet trolley with the screen printing loader, a 6-axis or 4-axis robotic arm design may be used.U型架取放板采用传统翻转取放板设计。丝印投板机一体式托盘台车取放板可采用六轴或四轴机械手方式。 - Except for the pre-curing unloader, which is designed with 3 empty U-Frame storage positions, 6 board storage positions, and 1 production position according to the design, all other U-Frame stations shall be designed with 2 empty U-Frame storage positions, 2 board storage positions, and 1 production position according to the design.除预固化收板机工位数量按设计3个空U型架暂存位，6个板件暂存位和1个生产工位外，其余U型架工位按设计2个空U型架暂存位，2个板件暂存位和1个生产工位设计。 | 取放邏輯滿足需求，工位設計以實際方案為準 | On-site confirmation 现场检查 |  |

- Process flow technical parameters工艺流程技术参数

| No 序号 | Process Flow 工艺流程 | Technical parameters 技术参数 | Supplier’s capability 供方能力 | Acceptance method 验收方法 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- |
| 1 | Working Mode 工作模式 | Loading: After retrieving boards from the U-frame rack or pallet, place them on the loader’s conveyor line; the boards shall be conveyed after board alignment positioning. 投板：从U型板架或托盘取板后放置在投板机传输线上，通过拍板定位后输送板件。 | 滿足需求 | Reviewed during 3D design review/On-site confirmation 3D会审时检查/现场检查 |  |
|  |  | Unloading: After boards enter the unloader’s conveyor line from the main line, retrieve the boards using the board alignment positioning method and place them into the U-frame. 收板：板件从主线进入收板机传输线后，使用拍板定位方式进行取板，放入U型架。 |  |  |  |
| 2 | Working Mode 工作模式 | - Loading: Equipped with upward-ejecting, center alignment, and sequential alignment functions.For the screen printing loader, the board alignment positioning method shall not be adopted when the contact surface between the board and the roller is an ultra-roughened surface. 投板：具备上顶，中心拍板、序列拍板功能。丝印投板机板件与滚轮接触面为超粗化面时不可采用拍板定位方式。 - Unloading: Equipped with upward-ejecting and center alignment functions.收板：具备上顶，中心拍板功能。 - Supports through-beam photoelectric or retro-reflective photoelectric alignment modes, as well as size-based alignment mode; the size can be set.​Minimum alignable board size: 400mm (for boards smaller than the minimum specification size).​Servo motors or stepper motors shall be adopted; when aligning 0.3mm copper-clad thin boards, no warping shall occur. An alignment structure with guide pillars shall be used.支持对射光电或回归反射光电拍板和尺寸拍板模式，尺寸可设定。最小拍板尺寸能力400mm（小于规格的最小板）。采用伺服或步进马达，0.3mm含铜薄板拍板不拱起，使用导引柱形式的拍板结构。 | 投板：具备上顶，中心拍板、序列拍板功能。丝印投板机僅從U架取出后傳送進主線，無定位功能。 收板：具备上顶，中心拍板功能 機構ok待電控回復sensor相關信息 | On-site confirmation 现场检查 |  |
| 3 | Board Handling Positioning Accuracy 收放板定位精度 | Board Alignment Precision: ≤ ±1mm. 拍板精度≤±1mm。 | 滿足需求 | On-site confirmation 现场检查 |  |
| 4 | Docking Mode 对接模式 | Manual docking 人工对接。 | 滿足需求 | On-site confirmation 现场检查 |  |
| 5 | Board Retrieval 取板 | - Each nozzle shall be equipped with an independent vacuum generator. Nozzles shall be compatible with different board sizes and allow adjustment of suction point positions.每个吸嘴采用独立真空发生器，吸嘴需要根据不同板件尺寸兼容使用，吸持点位置可调。 | 以實際方案為準 | On-site confirmation 现场检查 |  |
|  |  | - Equipped with vibration function, air-blowing function, and other functions to prevent boards from sticking.有抖动功能/吹气等功能，防止板件粘紧。 |  |  |  |
|  |  | - The vibration function shall be controlled by two groups of solenoid valves, with independent control for each group.抖动功能分两组电磁阀，独立控制。 |  |  |  |
|  |  | - Chips shall be prevented for nozzles, guide rods, nozzle holders, and board clamping mechanisms.吸嘴、导杆及吸嘴架、夹板机构要防止掉屑。 |  |  |  |
|  |  | - Automatic board drop detection and alarm function掉板自动检测及报警功能。 |  |  |  |
| 6 | Board Recognition板件识别 | Sensors can achieve 100% stable identification传感器能100%稳定判别。 |  | On-site confirmation 现场检查 |  |
| 7 | Board Rack Workstation板架工位 | For the Board Rack Detection in the workstation, through-beam sensing is adopted. 板架检知采用对射感应 | 滿足需求 | On-site confirmation 现场检查 |  |

- Equipment technical requirements设备技术要求

- Equipment basic requirements设备基本要求

| No 序号 | Item 项目 | Technical requirements 技术要求 | Actual design 实际设计 | Acceptance method 验收方法 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- |
| 1 | Place of origin of equipment 设备产地 | Not specified, please provide description 不指定，提供说明 | 廣東東莞 | / |  |
| 2 | Equipment length 设备长度 | Provide description 提供说明 | 以單機業務圖面為準 | On-site confirmation 现场检查 |  |
| 3 | Equipment length 设备宽度 | ≤2500mm | 以單機業務圖面為準 | On-site confirmation 现场检查 |  |
| 4 | Equipment height 设备高度 | ≤2600mm | 以單機業務圖面為準 | On-site confirmation 现场检查 |  |
| 5 | Net height from the bottom of the equipment to the ground 设备底部净高 | / | 100mm | / | The height of the space below the pipe 管道下方空间高度 |
| 6 | The biggest part that could not be disassembled 最大不可拆部件 | Width≤ 2400mm Depth≤3200mm Height ≤2400mm 宽度≤2400mm 深度≤3200mm 高度≤2600mm | 滿足需求 | On-site confirmation 现场检查 |  |
| 7 | Equipment weight 设备重量 | / | 3T | On-site confirmation 现场检查 |  |
| 8 | The heaviest part that could not be disassembled 最重不可拆部件 | ≤2T | 滿足需求 | On-site confirmation 现场检查 |  |
| 9 | Load-bearing of ground 地面承重 | ≤1.0T/m2 | 滿足需求 | On-site confirmation 现场检查 |  |
| 10 | Exterior color of equipment 设备外观颜色 | Use transparent materials for the exterior cover 外观盖板尽量使用透明材质 | 滿足需求 | On-site confirmation 现场检查 | 蓋板材質為10T透明PVC |
| 11 | Ambient temperature requirement 环境温度要求 | Room temperature 常温 | 滿足需求 | On-site confirmation 现场检查 |  |
| 12 | Cleanliness Requirement 洁净度要求* | - Reserve an installation position for FFU above the conveyor输送上方预留FFU安装位置。 - Equip the cover plate with a sealing strip盖板设置密封条。 - Design dust collection boxes at the bottom of all moving parts above传送上方的所有运动部件，底部设计集尘盒。 - Use anti-fall debris suction rod assemblies to prevent debris falling, and use non-marking suction nozzles.采用防掉屑吸盘吸杆组，防止掉屑，采用无印迹吸嘴。 - The parts in contact with the panels are not prone to aging, delamination, or chipping, and will not scratch or damage the panels.与板件接触部门不容易老化、脱胶、掉屑、不会擦花、压伤板件。 - There may be wear and air leakage at the cylinder guide rods, which should avoid contaminating the panels.气缸导杆处会有磨损及漏气情况，需避免对板件造成污染。 - The transmission protective cover is made of transparent material传动防护盖使用透明材质。 - Other requirements refer to the " Cleanliness Design Requirements for Automation Equipment"其它要求参照《自动化设备洁净设计要求》 | 1、六軸類設備具備此功能，其它無此功能 2、滿足需求 3、滿足需求 4、滿足需求 5、滿足需求 6、滿足需求 7、滿足需求 8、非六軸類設備部分規格不適用 | On-site confirmation 现场检查 |  |
| 13 | Illuminance Requirement 照度要求 | / |  | / |  |
| 14 | Noise related requirement 噪音要求 | Centralized silenced exhaust for compressed air 压缩气集中消声排放 | 滿足需求 | On-site confirmation 现场检查 | At 2 m from the noise source during operation 工作时距噪音源 2m 处 |
| 15 | Quantity of boards 板件数量 | Accurate counting of loaded/unloaded boards; identification of various abnormalities.精确计算投/收板数量，能够识别各种异常情况 |  | On-site confirmation 现场检查 |  |
| 16 | Roller 滚轮 | - Provide materials such as PP, HPEand NPP for selection in different processes提供PP、HPE、NPP等材质供不同工序选择。 - Must not scratch the board surface; thin-board passage shall be fully ensured. Indicate material on the right and don’t use PU rollers 不得擦花板件，且要充分考虑薄板通过性。在右侧写明材质, 不使用PU滚轮。 | 1、依2025/11/20提供台賬參數選擇滾輪材質。 2、依客戶要求選擇滾輪材質，保準滾輪本身無毛刺銳邊，無法保證完全不劃傷板面。 | On-site confirmation 现场检查 |  |
| 17 | Conveyor Drive Mode 输送驱动方式* | Magnetic non-contact drive 磁力无接触式驱动 | 滿足需求 | On-site confirmation 现场检查 |  |
| 18 | Air hose 气管 | Indicate material on the right; do not use PU air tubing 在右侧写明材质，不使用PU气管。 |  | On-site confirmation 现场检查 |  |
| 19 | Inductive Probe 感应探头* | Compatible with copper, black substrates, ink, ultra-roughened surfaces. 可适用铜、黑色基材、油墨、超粗化表面。 |  | On-site confirmation 现场检查 |  |
| 20 | Chip Prevention Device 防掉屑装置* | The conveying device shall adopt a servo motor; the use of air cylinders is prohibited 传送装置采用伺服电机，避免使用气缸。 | 傳送裝置使用馬達，固定高度如舉升、台車扣鎖使用氣缸。 | Reviewed during 3D design review/On-site confirmation 3D会审时检查/现场检查 |  |
| 21 | Double-Sheet Detection Function 双片检测功能* | The board loader shall be equipped with a double-sheet detection function 投板机需配置双片检测功能。 | 單U架設備無兩片檢知，U架及L架共用有此功能。 | Reviewed during 3D design review/On-site confirmation 3D会审时检查/现场检查 |  |
| 22 | Wiring Signal Docking 联线讯号对接 | Reserved interlocking points for front and rear machines shall be provided for wiring signal docking 预留前后机连锁点位 |  | On-site confirmation现场检查 |  |
| 23 | Design Review 设计会审 | Upon the seller’s completion of planning, design and subsequent 3D modeling (post - BOM release, pre - manufacturing), a joint design review with the buyer is required to jointly verify the design details for information alignment.卖方在完成规划设计后，3D设计后（下BOM后制造前），需与买方开展设计会审，共同确认设计细节，以便信息同步 | 滿足需求 | Design Review 设计会审 |  |
| 24 | Equipment design drawing 设计图纸 | Provide CAD drawings in DWG format and 3D models in STEP format. 提供DWG格式CAD图纸和STEP格式3D模型 |  | On-site confirmation 现场检查 |  |

- Basic requirements of energy能源基本要求

| No 序号 | Item 项目 | Technical specification 技术指标 | Actual Specifications 实际规格 | Acceptance Method 验收方法 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- |
| 1 | Rated power 额定功率 | Data to be provided by the supplier 由供方提供数据 | 以實際方案為準 | On-site confirmation 现场检查 |  |
| 2 | Air switch 空气开关 | Data to be provided by the supplier 由供方提供数据 |  | On-site confirmation 现场检查 |  |
| 3 | Voltage (V/Hz) 电压(V/Hz) | 3P/380V 50Hz |  | On-site confirmation 现场检查 |  |
| 4 | Current 电流 | Data to be provided by the supplier 由供方提供数据 |  | On-site confirmation 现场检查 |  |
| 5 | Compressed air pressure 压缩空气压力 | ≤6Kg/cm2 | 6Kg/cm2 | On-site confirmation 现场检查 |  |
| 6 | Compressed air flow 压缩空气流量 | LPM, data to be provided by the supplier LPM，由供方提供数据 |  | On-site confirmation 现场检查 |  |
| 7 | Compressed air pipe diameter 压缩空气管径 | inch/mm, data to be provided by the supplier inch/mm inch/mm，由供方提供数据 | 16mm | On-site confirmation 现场检查 |  |
| 8 | Energy consumption 能耗 | The supplier lists out the detailed energy saving and consumption reduction measures and effects 供方详细列出节能降耗的措施及效果 |  | On-site confirmation 现场检查 |  |

- Basic requirements for electrics and accessories电气和配件基本要求

| No 序号 | Item 项目 | Technical specification 技术要求 | Actual design 实际设计 | Acceptance Method 验收方法 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- |
| 1 | Electric control box design 电控箱设计 | Independent electric control box or online electric control box, the supplier provides the design独立电控箱或线上电控箱，供方提供设计。 | 以實際方案為準 | On-site confirmation 现场检查 |  |
| 2 | Electric control box incoming line电控箱进线 | Bottom in and bottom out or side in and bottom out, all junction box covers and electric box outlets are with sealing 下进下出或侧进下出,所有接线盒盖子和电箱出口带密封。 |  | On-site confirmation 现场检查 |  |
| 3 | Temperature requirements inside the electric control box 电控箱内环境温 度要求 | ≤40℃, the temperature of the components inside the electric control box is less than 60℃ during operation ≤40℃，运行时电控箱内元器件表面温度小于 60℃ |  | On-site confirmation 现场检查 |  |
| 4 | Air supply of electric control box 电控箱送风 | Positive pressure design, Sanyo /nmb/sunon/ Delta /avc/foxcon 正压设计，三洋/nmb/sunon/台达/avc/foxcon |  | On-site confirmation 现场检查 |  |
| 5 | PLC* | - Mitsubishi Electric FX-5U, Q or R series;program must remain unencrypted and include comments三菱FX-5U、Q或R系列，且程序不可加密，带点位注释。三菱FX-5U、Q或R系列，且程序不可加密，带点位注释。 - Each PLC shall reserve at least one Ethernet interface for connection to the buyer’s server每个PLC需预留至少一个以太网接口用于供买方服务器连接。 - Communication ports must be open to allow data reading and writing. The communication data format shall be configured in accordance with the template provided by the data acquisition system. The interactive data must be configured with an independent data DB block; this content shall be separated from the DB blocks related to the equipment control logic to ensure the stable operation of the equipment.通讯端口必须开放，可供读写数据。通讯的 数据格式，需要按照数据采集系统提供模板进行配置。交互的数据必须配置独立的数据DB块；该内容与设备控制逻辑相关DB块分离，以保证设备运行稳定性。 |  | On-site confirmation 现场检查 |  |
| 6 | PC | - On the HMI side, design mounting positions for a 22-inch display and standard keyboard-mouse set.在HMI侧设计22寸显示器、标准键鼠的安装位。 - Reserve placement space for a small industrial computer host, as well as a three-pin power socket and an air switch.预留小型工业电脑主机的放置空间和三脚电源插座及空开。 |  | On-site confirmation 现场检查 |  |
| 7 | Barcode(Panel) 读码（读板）* | - At the material inlet and outlet, reserve five rollers as the code-reading area. Within the 300mm × 300mm vertical space above the area, there shall be no pressure rollers or other components blocking, except for the reader mounting bracket. Opposed-mode or retro-reflective photoelectric sensors (installed at an angle) shall be installed at the front and rear of the code-reading area respectively. The sensors shall reliably, accurately and stably detect the presence or absence of boards, and their functions may be reused with existing sensors.出入料口预留五根滚轮作为读码区域，其上方300mm×300mm的立面空间内，除了读头安装架外，不得有压辊或其他部件遮挡，读码区域前后各要设置对射式或回归反射式光电传感器（倾斜安装），传感器要可靠、准确、稳定的感应到板件的有无，可与原有传感器功能复用。 - Design an angle-adjustable scanner mount to allow the reader to be adjusted within a range of ±60 degrees relative to the vertical of the PCB board surface.设计角度可调的扫码器安装件，使得读头能够垂直于PCB板面的正负60度范围进行调节。 - Reserve a junction box for the scanner’s signal cables and power cables, and reserve mounting holes in coordination with the code-reading implementation.预留读码器信号线和电源线接线盒，配合读码实施预留安装孔。 - The reader shall connect to the PLC via Ethernet to realize triggering and data reading. For the QR code data involved in code-reading data interaction, transmission and reporting to the system, at least 60 words of PLC addresses shall be reserved.读取器通过以太网链接PLC实现触发和数据读取,读码数据交互、传输、上报系统二维码数据对应PLC地址至少预留60字。 | 讀碼及讀頭安裝以實際方案為準 |  |  |
| 8 | Barcode(Carrier) 读码（载具）* | - Barcode reading rate requirement: 100%.读码率要求100% - The reader shall connect to the PLC via Ethernet to realize triggering and data reading.读取器通过以太网链接PLC实现触发和数据读取 - The PLC shall bind and unbind the board code-reading data and carrier data, upload the data to the EAP in real time, and leave a binding log on the PC.由PLC将板件读码数据和载具数据绑定和解绑，实时上传EAP且在PC留下绑定日志。 - The buyer’s server shall read and store the carrier data in real time, and write the data into the local code-reading log.买方服务器实时读取存储载具数据，写入本地读码日志。 - The program for server log storage shall be completed by the supplier, and the log storage format shall be confirmed with buyer.服务器存储日志的程序由供应商完成，日志存储形式需与买方确认。 - Configure barcode readers in quantities corresponding to the number of workstations; the supplier shall list the respective brands and models.配置对应工位数量的二维码读码器，需供方列出品牌和型号。 |  | On-site confirmation 现场检查 | Scenarios such as barcode damage, poor marking quality (resulting in 15% or more missing barcode modules), severe oxidation (where optical methods cannot optimize or resolve the issue), and manual operation interference shall not be included in the code reading rate statistics.二维码破损、打标质量不佳等导致二维码孔点缺失达到15%、严重氧化，光学方法无法优化和解决的、人工操作干扰的不包含在读码率统计中 |
| 9 | Barcode Scanners扫码枪 | Equip with wired barcode scanners and design mounting brackets for them. 配备有线扫码枪及设计安装支架 | 滿足需求 | On-site confirmation 现场检查 |  |
| 10 | Frequency converter, servo system 变频器、伺服系统 | Mitsubishi、Inovance、Yaskawa三菱、汇川、安川 | 台達/三菱 | On-site confirmation 现场检查 |  |
| 11 | Temperature Controller 温控器 | / | 無 | On-site confirmation 现场检查 |  |
| 12 | Circuit breaker (motor),leakage protection 断路器（马达）、漏电保护 | Fuji /Schneider /ABB /Mitsubishi 富士/施耐德/ABB/三菱 |  | On-site confirmation 现场检查 |  |
| 13 | HMI 人机界面* | Proface ET/ST 6000 Series (equipped with protective covers) Proface ET/ST 6000系列，带保护盖; |  | On-site confirmation 现场检查 |  |
| 14 | Ethernet Switch 交换机* | - Industrial-grade with 100/1000Mbps bandwidth 100/1000Mbps带宽，工业级别。 - Installed inside the equipment electrical control cabinet; both switches and network cables shall be clearly labeled设置在设备电控柜内部，包括交换机和网线都要有明确标识。 - Separate networking for inter-device communication and external communication; different switches shall be used for each设备之间的组网与设备对外的组网分开，并采用不同交换机。 - Reserve more than 4 spare ports after internal networking内部联网后仍预留4个以上网口空余。 - Use Cat6 Ethernet cables for internal networking内部网线使用六类网线组网。 |  | On-site confirmation 现场检查 |  |
| 15 | Electrical machinery / blower 电机/风机 | Not specified, to be filled in by the supplier 不指定，由供方填写 | 台达/凯福/三菱 | On-site confirmation 现场检查 |  |
| 16 | Light source 灯源灯源 | LED |  | On-site confirmation 现场检查 |  |
| 17 | Operation switch, indicator light 操作开关、指示灯 | Idec, Fuji, Schneider Idec，富士，Schneider |  | On-site confirmation 现场检查 |  |
| 18 | Relay 继电器 | Omron /Schneider Omron/施耐德 |  | On-site confirmation 现场检查 |  |
| 19 | Photoelectric sensor 光电传感器 | Must be waterproof, high reliability, low fasle alarm; Not specified, the supplier to list out the brand/model需要可防水，高可靠，低误报警。不指定，需供方列出品牌/型号 |  | On-site confirmation 现场检查 |  |
| 20 | Proximity switch 接近开关 | Not specified, to be filled in by the supplier 不指定，由供方填写 |  | On-site confirmation 现场检查 |  |
| 21 | Static eliminator 静电消除器 | - The board unloader shall be equipped with a static eliminator收板机需配置静电消除器 - Not specified, to be filled in by the supplier不指定品牌，由供方填写 | 收板機搭配靜電消除器，品牌KESD | On-site confirmation 现场检查 |  |
| 22 | Solenoid valve 电磁阀 | Not specified, to be filled in by the supplier 不指定，由供方填写 | SMC | On-site confirmation 现场检查 |  |
| 23 | Contactor 接触器 | Not specified, to be filled in by the supplier 不指定，由供方填写 |  | On-site confirmation 现场检查 |  |
| 24 | Profile 型材 | Not specified, to be filled in by the supplier 不指定，由供方填写 | SAA定制 | On-site confirmation 现场检查 |  |
| 25 | Bearing 轴承 | Not specified, to be filled in by the supplier 不指定，由供方填写 | 台灣品牌JWTC | On-site confirmation 现场检查 |  |
| 26 | Pressure Meter 压力表 | Digital display, read - write enabled 数显，可读写 | 指針式，可讀寫 | On-site confirmation 现场检查 |  |
| 27 | Robot 机械手 | - Preferred Selection: Inovance、Rokae、Hik 优选：汇川、珞石、海康 - Provide cleanroom certification documents or test reports corresponding to the selected models.提对应型号的洁净度认证资料或者测试报告 | 珞石 潔淨度認證見附件資料 | On-site confirmation 现场检查 |  |

Note: The items in the above table can be added by the supplier according to the actual situation to present the technical advantages or special requirements of the supplier. The same category components of the entire line should be of the same brand, unless necessary.

备注：上表项目请供方根据实际情况增加，以表达供方的技术优势或特殊要求。整线的同类配件，除必要外，应采用同一品牌。

- Requirements of electric cabinet电柜要求

| No 序号 | Item 项目 | Specification 要求 | Actual design 实际设计 | Acceptance Method 验收方法 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- |
| 1 | Power supply 电源 | AC 3P (3-wire/4-wire/5-wire), 380V@50Hz or AC 1P 220V@50Hz AC 3P（3线/4线/5线） 380V@50Hz 或AC 1P 220V@50Hz |  | On-site confirmation 现场检查 | if the on-site power supply does not meet the voltage/frequency requirements ,the transformer shall be self-provided.否则自备变压器 |
|  |  | Equipped with main power incoming line terminals 配置主电源入线端子 |  | On-site confirmation 现场检查 |  |
|  |  | Each cabinet needs to be equipped with a power switch 每一个电柜需配设一电源开关 |  | On-site confirmation 现场检查 |  |
| 2 | Material 材质 | In general area, the cabinet can be made of Q235 material with thickness of 1.6mm, with white powder baking paint一般区域采Q235厚度1.6mm材质粉体烤漆白色 | 一般区域采Q235厚度1.5mm材质粉体烤漆白色 | On-site confirmation 现场检查 |  |
|  |  | In acid/alkaline area, the cabinet can be made of SUS304 material with thickness of 1.6mm and coated with epoxy resin paint, or 10mm PVC, PP material. 酸碱性区域采厚度1.6mm SUS304涂环氧树脂漆或10mm PVC、PP材质 | 酸堿区域采Q235厚度1.5mm材质粉体烤漆白色 | On-site confirmation 现场检查 |  |
|  |  | In clean room, SUS304 material shall be used,thickness ≥1.0mm无尘室采用SUS304材质, 厚度1.0mm | 滿足需求 | On-site confirmation 现场检查 |  |
|  |  | In outdoor area, 2.0mm SS 41 material can be used, with white powder baking paint室外区域采厚度2.0mm材质采SS 41粉体烤漆白色 | 室外区域采Q235厚度1.5mm材质粉体烤漆白色 | On-site confirmation 现场检查 |  |
| 3 | Labeling 标识 | Each switch and indicator on the panel of the cabinet should be labeled 电柜面板各开关及指示灯均需标示 |  | On-site confirmation 现场检查 |  |
|  |  | Inside the cabinet, between the components, need to be pasted labels nearby to indicate its usage corresponding to drawings电柜内部的元件，需在附近粘贴与图纸对应的标识，指明其作用 |  | On-site confirmation 现场检查 |  |
| 4 | Wiring 配线 | It is not allowed to open holes on the top of the cabinet, all wiring needs to be laid along the cable rack. The wire enters in from the hole opened on the side of cabinet (protection sleeve shall be provided for the wires, and the wire can not contact the cabinet directly), there must be no any gas / liquid pipelines above the cabinet. 不可由上方开孔,所有配线需沿线架,在箱体侧方开孔接入(需有电线保护套不可让绝缘层直接与金属外壳接触), 柜体上方不得有任何气体/液体管路经过 |  | On-site confirmation 现场检查 |  |
|  |  | The wiring in the cabinet should be covered in cable trays and jump connection is allowed 柜中的配线，应使用线槽收纳，不得跳接 |  | On-site confirmation 现场检查 |  |
| 5 | Track轨道 | All electrical components inside the cabinet are fixed with aluminum tracks, and 15% of the track space needs to be reserved for future expansion and connection 箱内电器组件均使用铝轨道固定,并需预留15%的轨道空间,以利日后扩充连接 |  | On-site confirmation 现场检查 |  |
| 6 | Lighting 照明 | Light is installed inside the cabinet, which lights up when the door is opened or the external switch is turned on 柜内设置照明灯，门开时及外部开关时，柜内点亮 |  | On-site confirmation 现场检查 |  |
| 7 | Socket 插座 | The cabinet is equipped with 220VAC sockets(follows Thai standard), at least one 2P and one 3P柜内配设有220VAC插座（泰标），2孔+3孔至少各一 |  | On-site confirmation 现场检查 |  |
| 8 | Heat Dissipation 散热 | The electrical cabinet needs to be provided with heat dissipation function, The air supplying and ventilation ports are designed, with fans and simple filtering devices installed. The electric appliances, such as fan, independent fuse shall be installed 电柜需有散热功能，设计进风口抽风口，安装风扇及简易过滤装置,风扇等用电器具单独加装保险丝 |  | On-site confirmation 现场检查 |  |
| 9 | Base 底座 | The base of electrical cabinet must be made of SUS angle steel or square steel pipe, to keep the cabinet 10cm above the ground and firmly fixed 电柜须以SUS角钢或方形钢管做底座且与地面保持10cm架空并固定妥当 |  | On-site confirmation 现场检查 |  |
| 10 | Temperature monitoring温度监控 | A temperature display should be installed outside the electrical cabinet to monitor the temperature inside the cabinet, and the alarm is triggered for over-temperature 电柜外需装置温度显示器,以监控箱内温度 |  | On-site confirmation 现场检查 |  |

- Power distribution requirements配电要求

| No 序号 | Item 项目 | Specification 要求 | Actual design 实际设计 | Acceptance Method 验收方法 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- |
| 1 | Cable type 电缆类型 | The color and diameter of the power supply cable should be in accordance with the relevant regulations of Thailand, the wiring outside the cabinet should use multi-core cable, and the signal line should use shielded cable and well grounded. RVVT flexible cable should be used for trailing chain, and flame-retardant cable should be used for water pumps, motors and heaters 电源线色及线径需符合法规,电柜以外的配线使用多芯电缆线,讯号线需使用屏蔽电缆并良好接地。拖链处电缆需要采用 RVVT软性电缆,水泵、电机及加热器电缆需采用阻燃电缆。 |  | On-site confirmation 现场检查 |  |
| 2 | Color 颜色 | Control power supply is 24V DC, line color: blue.Phase line color: U yellow, V green, W red Grounding wire color: yellow and green 控制电源一律为24V DC线色:蓝色。 相线颜色：U黄，V绿，W红 地线颜色：黄绿交替 |  | On-site confirmation 现场检查 |  |
| 3 | Cable tray 线槽 | The wiring in the cabinet is provided with PVC cable trays 柜内配线以PVC线槽配设 |  | On-site confirmation 现场检查 |  |
|  |  | If there are more than 5 wires for connecting the cabinet with the equipment, please use cable rack or cable tray, of which the material should be selected according to the specification of the switch box 电柜与机台联机线数达5条以上时请以电缆架或线槽施工,材质依开关箱规范配置。 |  | On-site confirmation 现场检查 |  |
| 4 | I/O port 出入口 | All power lines and control lines must enter and go out from the side of the electrical cabinet, with enclosed cable trays and movable covers, and separated 电力线及控制线一律由电柜侧边出入,以密闭线槽加活动盖并各别区隔。 |  | On-site confirmation 现场检查 |  |
| 5 | Labeling 标识 | The label of each component is printed and attached to the cable tray cover (the label must be consistent with the drawing). 各组件标识打印贴附于线槽盖(标识须与图纸一致)。 |  | On-site confirmation 现场检查 |  |
|  |  | The control lines are numbered according to the wire numbers on the circuit diagram (marked on the protective sleeves). 控制线依电路图线号进行编号(标示于护套上)。 |  | On-site confirmation 现场检查 |  |
| 6 | Terminal 端子 | Each wire should be equipped with appropriate wiring terminals, and the power lines should be connected by crimping with hexagonal hydraulic terminal plier, and added with protection cover plates and wire sleeves 各导线需配设适当接线端子,动力线需以六角油压端子钳压接,并加装保护盖板及线套。 |  | On-site confirmation 现场检查 |  |
|  |  | Wire connections must be sealed with terminal blocks and boxes, and cannot be directly connected. 电线连接须用端子排及盒子密封,不可直接对接。 |  | On-site confirmation 现场检查 |  |
|  |  | Provide interlock I/O signals and wiring for upstream/downstream machine connection. 提供前后机联机接点及接线。 |  | On-site confirmation 现场检查 |  |
|  |  | Main wiring terminal block is provided for equipment grounding 提供设备接地总接线端子台。 |  | On-site confirmation 现场检查 |  |
| 7 | Set up 架设 | If the equipment and the electrical cabinet are separated, the connection cables are set up elevated. The main cable trays (power lines, signal lines) height is over 2.6 meters 机台与电柜如为分离式，电缆架设方式以架高方式衔接配置。主线槽(电力线,信号线)架高2.6米以上 |  | On-site confirmation 现场检查 |  |
|  |  | All electrical lines shall not be laid on the ground, with the principle of being at least 10cm above the ground; The wires of auxiliary tank and PUMP along the module must not be placed on the ground and must be raised at a height of more than 50cm. 所有电线管路不落地,以离地十公分以上为原则；槽边副槽及PUMP电线,不可落地,架高50公分以上。 |  | On-site confirmation 现场检查 |  |
|  |  | AC and DC cable trays need to be separated to prevent interference and cause false signals. 交流直流线槽需分开布线,防止干扰误讯号。 |  | On-site confirmation 现场检查 |  |
| 8 | Fixation 固定 | The wiring outside the junction box outside the electrical cabinet shall use cables with PVC or PE winding pipes. The opening of the junction box is fixed with waterproof connectors. 电柜外接线箱外的配线使用电缆线并用PVC、PE缠绕管配管，线盒开口处使用防水接线头固定。 |  | On-site confirmation 现场检查 |  |
|  |  | Bending resistant flexible cables with trailing chain or PVC, PE winding pipes should be used for the moving parts. 活动部位需使用耐弯曲的柔性电缆加坦克链或PVC、PE缠绕管配管。 |  | On-site confirmation 现场检查 |  |

- Safety device安全装置

| No 序号 | Item 项目 | Technical Specification 技术要求 | Actual Requirements 实际要求 | Acceptance Method 验收方法 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- |
| 1 | Rotating mechanism 转动机构 | Install the protective cover 安装护罩 |  | On-site confirmation 现场检查 | Gear /pulley /brush wheel parts 齿轮/皮带轮/刷轮部位 |
| 2 | Equipment operation protection 设备运行保护 | Photoelectric sensing and alarm、The equipment stops upon door opening 光电感应报警、开门后停止动作 |  | On-site confirmation 现场检查 |  |
| 3 | Leakage protection of electrical box 电箱漏电保护 | All electrical cabinet door devices are connected to the ground wire 所有电柜门装置 连接地线 |  | On-site confirmation 现场检查 |  |
| 4 | Equipment alarm system 设备报警系统 | Tri color LED light三色LED 灯 |  | On-site confirmation 现场检查 | Installed in two positions, the beginning and the end of the line 拉头、尾 2 处 |
|  |  | Alarm sound 报警声音 |  | On-site confirmation 现场检查 | Volume > 90 dB; Installed in two positions, the beginning and the end of the line 音量>90分贝，头、尾 2 处 |
| 5 | Emergency switch 紧急开关 | Equipped with emergency stop pull rope and reset button 配紧急停止拉绳和复位按钮 |  | On-site confirmation 现场检查 | Pay attention to designing emergency switches for areas with operation safety risks 注意设计有操作安全风险 区域的紧急开关 |
| 6 | Wiring Requirements接线要求 | All wires have no joints in between and are equipped with enclosed cable trays. The junction box is designed to be waterproof and splash proof 所有电线中间无 接头，配备密闭线槽，接线盒防水溅设计 |  | On-site confirmation 现场检查 | The color of cables for the same usage should be uniform, and the power line and signal line should be distinguished by color 同种用途电缆颜色统一， 电源线、信号线的颜色加以区分 |

- Automation Requirements自动化要求

| No序号 | Item 项目 | Technical specification 技术指标 | Supplier Capability 供应商能力 | Acceptance Method 验收方法 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- |
| 1 | PLC I/O | All PLC outputs must drive loads indirectly via interposing relays; direct load switching is not permitted.Every spare PLC I/O point is pre-wired to the terminal block PLC输出控制需以继电器做间接控制,不可直接带动负载，所有PLC备用I/O点均配线至端子台。 |  | On-site confirmation 现场检查 |  |
| 2 | Auto / Manual mode自动/手动功能 | Equipped with automatic /manual control mode具有自动/手动控制模式 |  | On-site confirmation 现场检查 |  |
| 3 | Board-Jam Alarm塞板报警 | Available. The supplier will provide an alarm solution有。供方提供报警方案。 |  | On-site confirmation 现场检查 |  |
| 4 | Power-saving Mode省电功能 | Energy-saving motor with auto-stop function – conveyor halts when no board is present具备节能模式，无板时停止输送等马达 |  | On-site confirmation 现场检查 |  |
| 5 | HMI 触摸屏 | Event logs and alarm information for HMI shall support power-off retention.HMI的事件记录、报警信息需断电保持。 |  | On-site confirmation 现场检查 |  |
| 6 | Operation 操作 | It is equipped with physical buttons including power switch, start, stop, reset, and emergency stop for ease of operation.具备电源开关、启动、停止、复位、急停等物理按钮，便于操作。 |  | On-site confirmation 现场检查 |  |
| 7 | Programs 程序 | The programs of the PLC, HMI, and PC shall not be encrypted; comments shall be provided for data collection purposes. PLC和HMI、PC不得加密，提供点位注释供数据采集使用。 |  | On-site confirmation 现场检查 |  |

- Intelligent Requirements智能化要求

- Basic requirements基本要求

| No 序号 | Item 项目 | Technical requirements 技术要求 | Supplier’s capability 供方能力 | Remarks 备注 |
| --- | --- | --- | --- | --- |
| 1 | General requirements for intelligence 智能化总体需求* | - The equipment supports the downloading of task queues by MES/EAP, the equipment verifies panel and material information with MES (if required by the equipment), and carry out control.设备支持可被MES/EAP下载任务，设备与MES校验板件信息并进行卡控。 - The equipment supports real-time status , alarm data collection ,barcode data of boards and carriers by MES/EAP.设备支持可被MES/EAP采集实时状态、实时报警数据、板件以及载具二维码数据。 - The equipment stops and doesn’t stop when MES/EAP triggers alarm, with message display.设备支持可被MES/EAP触发报警停机和报警不停机，并附带消息显示。 |  |  |
| 2 | Timing configuration 授时配置 | The equipment PLC, HMI and PC need to have a unified time, and connect to the supplier's timing server.设备PLC、HMI、PC需统一时间，连接供方授时服务器。 |  |  |
| 3 | Communication protocol 通信规范 | - Real time status, alarm, and process data can be communicated through OPC-DA, OPC-UA, or Mitsubishi MC protocol. The detailed explanation to the TAG and location shall be provided corresponding to the adopted protocol.实时状态、报警、过程数据，可通过OPC-DA、OPC-UA、三菱MC协议通信。提供所采用协议相关的TAG、点位详细对应说明。 - The data related to production task queue functions can be communicated through OPC-DA, OPC-UA, Mitsubishi MC protocol and other methods. The detailed explanations to the TAG, location, interface, data dictionary, etc. shall be provided corresponding to the adopted protocol.生产任务相关功能的数据，可通过OPC-DA、OPC-UA、三菱MC协议等方式通信。提供所采用协议相关的TAG、点位等详细对应说明。 - For the interaction specifications between MES/EAP and equipment, this specification describes the overall logic. The buyer shall provide interface specification documents such as "EAP System PLC Equipment Communication Specification" before implementing intelligent functions of the equipment, to explain the specific interaction interfaces. The buyer may update and adjust the documents as needed during the implementation process, and the supplier shall cooperate to complete it.对于MES/EAP与设备的交互规格，本规格说明的是整体逻辑。买方在设备实施智能化功能前提供《EAP系统PLC设备通讯规范》、等接口说明文档，对具体交互接口进行说明。买方在实施过程可能会按需对文档进行更新调整，供方需配合完成。 |  |  |

Equipment state设备状态

| No 序号 | Item 项目 | Technical specification 技术指标 | Supplier’s capability 供应商能力 | Remarks备注 |
| --- | --- | --- | --- | --- |
| 1 | Data requirements 数据要求 | - The status information provided by the equipment is not simply the three-color light status, but reflects the overall condition of the equipment, as shown in "State modeling".设备提供的状态信息并不是单纯的三色灯状态，而是体现设备整体状况，见“状态建模”。 - The equipment status can be collected by MES/EAP.设备状态支持被MES/EAP采集。 |  |  |
| 2 | State modeling状态建模 | No 序号; Type 类型; Description 说明. 1; Automatic operation自动运行; Production state in automatic mode, with panels on the production line自动模式下生产状态，产线有板. 2; Manual operation手动运行; Production state in manual mode, with panels on the production line手动动模式下生产状态，产线有板. 3; Standby待机; Standby mode, no panel on the production line待机状态，产线无板. 4; Stop停止; Stop state, no panel on the production line停止状态，产线无板. 5; Maintenance保养; Maintenance state, no panel on the production line保养状态，产线无板. 6; Running alarm运行报警; Alarm during operation运行时，报警产生. 7; Standby alarm待机报警; Alarm during standby mode待机时，报警产生. 8; Stop alarm停止报警; Alarm when stop停止时，报警产生. 9; Maintenance alarm保养报警; Alarm during maintenance保养时，报警产生. |  |  |

Equipment events and alarms设备事件和报警

| No 序号 | Item 项目 | Technical specification 技术指标 | Supplier’s capability 供应商能力 | Remarks备注 |
| --- | --- | --- | --- | --- |
| 1 | Event 事件 | Provide all event related interfaces or point addresses, tag names and corresponding descriptions.提供所有事件相关接口或者点位地址、Tag名称和对应说明。 |  |  |
| 2 | Alarm 报警 | Provide all alarm related interfaces or point addresses, tag names and corresponding descriptions.提供所有报警相关接口或者点位地址、Tag名称和对应说明。 |  |  |

Production Task生产任务

| No 序号 | Item 项目 | Technical specification 技术指标 | Supplier’s capability 供应商能力 | Remarks备注 |
| --- | --- | --- | --- | --- |
| 1 | Overview概述 | - The equipment needs to have production task queue for remote transmission by MES/EAP.设备需具备生产任务功能，用于MES/EAP远程下发。 - The specific interfaces refer to "EAP System PLC Equipment Communication Standard" provided by the buyer in advance.具体接口参考买方在实施前提供的《EAP系统PLC设备通讯规范》 |  |  |
| 2 | Parameter 参数 | - The parameters required for production tasks include: production number (material number), batch number, panel quantity, panel size and other production parameters生产任务所需要的参数包括：生产编号（料号）、批次号、板件数量、板件尺寸等生产参数。 |  |  |
| 3 | Interaction logic 交互逻辑 | The following is the general principle for the interaction between MES/EAP and the equipment production task queue function; The specific interface can be found in the document "EAP System PLC Equipment Communication Specification" provided by the buyer. 以下是MES/EAP与设备生产任务队列功能的交互总则；具体接口见届时买方提供的《EAP系统PLC设备通讯规范》文档。 - MES/EAP downloads production tasks to the equipment, and the equipment responds whether the download is successful.MES/EAP下载生产任务至设备，设备应答下载是否成功。 - The equipment shall compare the number of tasks in the queue with the actual production quantity, and automatically report to MES/EAP when the task at the current station is completed.设备根据队列数量和实际生产数量进行比对，设备当前工位任务完成时自动上报MES/EAP。 - The equipment shall be equipped with an alarm function for excess boards or missing boards, and a function to continue board feeding when there are excess boards.具备多板少板报警功能，具备多板继续投板功能。 - The equipment operation interface supports compulsory task completion: that is, forcing to finish the current task.设备操作界面支持强制完工：即强制结束当前任务。 |  |  |
| 4 | Information notification 信息通知 | The equipment supports receiving messages issued by MES/EAP as notifications during the interaction process.设备支持接收MES/EAP下发的信息，作为交互过程的通知。 |  |  |
| 5、 | Task Switching任务切换 | - The batch completion signal of the current task shall be transmitted to the downstream main line equipment.任务结批信号需传递给后机主线设备。 |  |  |

Control卡控

| No 序号 | Item 项目 | Technical specification 技术指标 | Supplier’s capability 供应商能力 | Remarks备注 |
| --- | --- | --- | --- | --- |
| 1 | Control function 卡控功能 | - The equipment can receive the control information from MES/EAP and return with two types of response depending on the control information types: alarm but can still input panel, alarm and can not input panel设备可接收MES/EAP的卡控信息，并根据卡控信息类型的不同，做出报警能进板、报警不能进板两种响应。 - The equipment supports receiving messages issued by MES/EAP as the notification for control设备支持接收MES/EAP下发的信息，作为卡控信息通知 - The equipment shall be equipped with independent switches for each type of interlock control; the corresponding interlock control types are described in the " Equipment Operation Plan".设备需要对每种卡控有独立开关。对应卡控类型在《设备运营案》中描述。 |  |  |

Barcode data reporting读码数据上报

| No 序号 | Item 项目 | Technical specification 技术指标 | Supplier’s capability 供应商能力 | Remarks备注 |
| --- | --- | --- | --- | --- |
| 1 | Data reporting 数据上报 | - After the equipment reads the board's barcode, it shall report the data to the EAP in real time设备读取到板件二维码后实时上报EAP。 - After the equipment reads the carrier's barcode, it shall report the data to the EAP in real time.设备读取到载具二维码后实时上报EAP。 - The equipment shall support the function of reporting the board's barcode and the carrier's barcode in a bound manner.支持板件二维码与载具二维码绑定上报功能。 |  |  |

- Specification and requirements of design设计规范要求

The entire equipment shall comply with the relevant industry standards of the country where Party A is located.整机需符合甲方所在国的相关行业标准。

- Documents of equipment设备资料

- Upon the completion of equipment installation, before the formal acceptance, Party B must provide at least one paper copy and the electronic file of maintenance manual and operation manual in English, Chinese and Thai, including the following content:设备安装完成后、正式验收之前，乙方必须提供英、中、泰文维修手册及操作手册至少一本并同时提供电子档，包含以下内容：

- Equipment description设备描述；

- Safety sign安全警示；

- Switch on/off sequence开关机顺序；

- Electromechanical operating principle操作机电原理；

- Operation parameter settings操作参数设定；

- Electromechanical calibration parameters and operating procedures机电校正参数及操作步骤；

- Electromechanical assembly drawings机电装配图；

- Recommended maintenance frequency, level and content, as well as the tools and materials to be used for maintenance建议的维护保养频率、级别和内容和所使用的工具、材料等；

- Troubleshooting instructions故障排除指示；

- Instructions and diagrams for disassembly and assembly of components部件拆装说明及示意图；

- Contact information of the service center and the warehouse location information of the spare parts服务中心的联系方法及备件仓存的位置；

- Spare parts list备件清单。

- A's requirements before the equipment installation in the factory, and the spare parts list shall be provided before the completion of equipment acceptance, the spare parts include consumables, preventive maintenance spare parts, long-lasting spare parts and etc. The spare parts list should contain the following information乙方应该按照甲方的要求在设备到厂安装前提供设备安装用的材料、备件以及工具等，在设备完成验收前提供以下要求的备件清单，备件包括耗材、预防维护用备件、长效备件等。备件清单应该包含以下的内容：

- Equipment manufacturer's part number, description and function设备制造商的备件编号、描述以及功能；

- The spare parts manufacturer code备件制造商的编号；

- The used quantity of spares parts per equipment备件在每台机的数量；

- Wear and tear cycles of the spare parts and recommended replacement frequency备件的损耗周期以及建议更换周期；

- Delivery lead time of spare parts备件供货周期；

- Define the category of spare parts, such as consumables, maintenance spare parts, long-lasting spare parts, etc. Consumables refer to the spare parts with the wear & tear cycle less than the free maintenance period of the equipment. For consumables, the reasonable wear & tear cycle shall be specified. If the actual wear & tear cycle of consumables is less than that defined by Party B, Party B is responsible for replacing the part free of charge and improving the service lift of the parts to reach the original defined wear and tear cycle界定备件的种类，如耗材、维护保养用备件、长效备件等，耗材指的是损耗周期小于设备免费维修周期的所有备件．对于耗材，需要指明其合理损耗周期，如果使用中发生耗材的实际损耗周期小于供方界定的损耗周期，供方负责免费更换并改善至原始界定的损耗周期。

- Service life agreement of key parts (if the service life is not reached, Party B shall replace or repair it free of charge)核心配件使用寿命约定（未达到使用寿命由供方免费更换或者维修）：

| NO 序号 | Spare parts description 配件名称 | Brand 品牌 | Specification and model 规格型号 | Service life 使用寿命 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- |
| 1 | Drive 传送 |  |  |  |  |
| 2 | Motor 马达 | 台達/凱福 | 以實際方案為準 | 2 |  |

- Spare parts and consumables delivery list配件及消耗品配送清单：

| NO 序号 | Spare parts description配件名称 | Brand 品牌 | Specification and model 规格型号 | QTY 数量 | Remarks 备注 |
| --- | --- | --- | --- | --- | --- |
| 1 | PLC module PLC 模块 |  |  |  |  |
| 2 | Frequency converter 变频器 |  |  |  |  |
| 3 | Sensor 传感器 |  |  |  |  |
| 4 | wearing parts易损件 |  |  |  |  |

- Frequency converter: At least one operation manual and its electronic file shall be provided变频器:操作说明书至少一本并同时提供电子档。

- 6.	At least one copy of PLC and HMI backup program or PC control system installation disk and operating software . PLC 及人机备份程序或 PC 控制系统安装盘和操作软件至少一本并同时提供电子档。

- At least one operation manual and its electronic file for each type of servo controller shall be provided各种伺服控制器的操作说明书至少一本并同时提供电子档。

- At least one operation manual and its electronic file for each type of temperature controller or sensor shall be provided 各种温度控制器或传感器操作说明书至少一本并同时提供电子档。

- Equipment installation and commissioning设备安装调试

- Before the equipment installation, Party B must provide clear information so that Party A can accurately determine all the external conditions required for the installation of equipment, including space, environment requirements, supporting facilities, installation tools and etc安装前，乙方必须提供清洁的资料以便甲方能够准确的判定设备安装所需要的一切外部条件， 包括空间、环境要求、配套设施、安装工具等。

- Party B is responsible for all internal installation, piping connection and commissioning of the equipment, If Party A’s support is needed, it needs to be explained in the initial information provided and confirmed in advance乙方负责设备的所有内部安装、管线接驳和调试等，如需甲方配合需要在事先提供并确认的初始资料中说明。

- Positioning of equipment: according to the equipment process layout and installation & construction drawings, the location of the foundation, equipment outline dimension and mutual spacing and etc. are marked for positioning, organize foundation construction and equipment moving in place. In addition to the principles of following the installation process layout, the positioning and installation of equipment should focus on the following points.设备的定位:按照设备工艺平面布置图及安装施工图，基础图的位置、设备轮廓尺寸及相互间距等要求划线定位，组织基础施工及设备搬运就位。设备的安装定位除安装工艺布置的原则外要重点考虑以下几点:

- The needs of product processing conditions shall be considered (including ambient temperature, dust, noise, light, vibration)应考虑适应产品加工条件的需要(包括环境温度．粉尘、噪声、光线、振动等)。

- It should be convenient for workpiece storage, transportation and cleaning of chips, and good for the set management in the workshop ("5S" management) and civilized production.应方便工件的存放、运输和切屑的清理，有利于车间的定置管理(“5S”管理)和文明生产。

- The external dimensions of equipment main body and the auxiliary devices, and the limit position of the moving parts shall be considered.应考虑设备主体，附属装置的外形尺寸及运动部件的极限位置。

- The equipment installation, maintenance and operation safety requirements shall be considered应满足设备安装、维护、操作安全的要求。

- The span of the plant, the width and height of the door must be considered必须考虑厂房的跨度、门的宽度和高度。

- The power supply and labor protection requirements should be considered应考虑动力供应和劳动保护的要求。

- All pipelines as well as supporting hangers are constructed in strict accordance with the corresponding specifications, with neat and orderly appearance所有管线以及支吊架严格按照相应的规范施工，外观整齐有序。

- After the installation is completed, Party B should designate technicians to follow up until Party A signs the equipment acceptance report安装完成后，乙方应该安排指定的技术人员跟进直至甲方签署设备验收报告。

- For the equipment specified in this tender, Party B will provide the overall plan drawings and technical parameters for equipment installation, and Party B will arrive at the equipment installation site within one week after receiving Party A's notification, and carry out the installation and commissioning of the whole equipment本次招标规定的设备，由乙方提供设备安装总体平面图纸与技术参数要求，乙方在收到甲方通知后一周内到达设备安装现场进行整机安装、调试；

- After the installation, commissioning and normal trial run, the equipment will be handed over to Party A for use, the cost is included in the contract price. Party B completes the installation and commissioning of the whole equipment within the agreed time安装、调试及试运行正常后交付甲方使用，其费用已包括在合同价内。乙方在约定时间内完成整机设备的安装调试工作；

- For all equipment and the accessories provided by Party B, Party B shall provide necessary assistance to support the relevant departments of China and Thailand for equipment inspection when needed. After the completion of the equipment installation, Party A organizes personnel to check and accept the quality, specifications, quantity and performance of the equipment in accordance with the contract requirements乙方提供的所有设备及其附件，需要时配合中国和泰国相关部门查验提供必要协助。在设备安装完成后由甲方组织人员对设备的质量、规格、数量及性能等方面按合同要求进行检查验收。

- Confirm if the installation and commissioning are successful确认安装调试是否成功；

- Check and confirm if the equipment performance as well as other quality and technical indicators meet the contract requirements确认设备性能及其它质量技术指标是否符合合同要求

- Acceptance method: based on the equipment purchasing contract, the equipment acceptance shall be carried out according to the equipment acceptance method specified in this technical specification验收方法：根据设备订购合同，按本技术规格书规定的设备验收方法验收。

- Training培训

- Party B is responsible for providing training to the equipment operators of Party A, and the required training costs shall be borne by Party B乙方负责对甲方的设备操作人员进行培训，所需培训费用由乙方承担。

- Party B must provide training to our operators, process engineers and equipment maintenance engineers, and training materials shall be provided so that authorized trainers of our company could train other employees, the training content should include at least the following乙方必须对我公司的操作员工、工艺工程师和设备维护工程师提供培训，并且提供培训资料以便公司内部授权的培训人员可以继续培训其他员工，培训内容应该至少包括以下内容：

- Equipment description设备描述；

- Safety warnings安全警示；

- Installation, turn on/off operation安装、开关操作；

- Detailed Software Interaction Document详细软件交互文档。

- The electrical and mechanical operation principles操作电气以及机械原理说明；

- Description of the relevant indicator signals相关指示信号说明；

- Mechanism assembling and disassembling instructions机构装、卸指示；

- Instructions for peripheral water, electricity, wind and gas外围水、电、风、气说明；

- Maintenance, diagnosing methods and practices维修、诊断方法和实践；

- Maintenance requirements and practices维护保养要求和实践。

- Equipment acceptance设备验收

The final acceptance of the equipment shall involve all content specified in the contract, technical specifications and other binding documents, including the completeness of equipment modules and documents, equipment stability and equipment process capability criteria.设备的最终验收需要包括所有在合同，技术规范书和其他具有约束效力的文件规定的所有内容，包括设备模块完整性、资料完整性、设备稳定性、设备工艺能力标准等。

  - Confirm that all equipment configurations and functions meet the equipment description确认设备的所有配置、功能符合设备描述；

  - Confirm that all control systems, control signals are correct and effective确认所有控制系统、控制信号正确并有效；

  - Confirm that the equipment pipeline connections comply with the regulations of our company确认设备管线接驳符合公司规定；

  - Complete the training and provide training materials as agreed in the training program按照培训项目约定完成相关培训及资料提供；

  - Reach the related conditions as agreed in equipment stability program按照设备稳定性项目约定符合相关条件；

  - Reach the relevant conditions based on the agreed description of equipment functions (modules)按对照设备功能(模块)描述设备约定符合相关条件；

  - The test results reach the relevant conditions as agreed in the process specifications按照工艺技术规范项目约定试验结果满足相关条件；

  - Provide all related documents according to requirements for documentation project按照文档项目规定提供所有资料。

- Stability related indicators稳定性指标

MTBF ≥ 1400 hours

MTBF = （Scheduled time - Unscheduled Downtime）/ Number of failures

MTTR≤ 2 hours

MTTR = (Unscheduled Downtime – Wait Time )/Number of failures

MTBF and MTTR are calculated based on the following data:

MTBF 和 MTTR 计算依据如下:

Scheduled Time: the time scheduled for equipment production = 22.0hours/day × 7 days/week (360days/year) = 7920 hours/year

Scheduled Time ： 设备计划用于生产的时间= 22.0 hours/day × 7 days/week (360 days/year)

= 7920 hours/year

Failure: Any reason that causes the equipment to be unable to continue production, including equipment failure or potential quality risks that exceed the requirements of the equipment specifications

Failure ：任何导致设备无法继续生产的原因，包括设备故障或潜在超出设备工艺技术规范要求的品质隐患。

Down Time: The time that the equipment stops due to the above reasons

Down Time ： 由于上述原因导致的停机。

Wait Time: The time from the equipment shutdown due to failures till our company formally notifying the supplier. Formal notification includes but not limited to telephone, fax, e-mail, etc

Wait Time ：设备在发生故障停机后，到我公司正式通知到供方为止的时间。正式通知包括但不局限于电话、传真、电子邮件等。

- Warranty period and technical support service保修期及技术服务

- The warranty period is one year, calculated from the date that Party A signs for acceptance.

保修期为一年，自甲方验收签字验收之日起计算。

- After the equipment shuts down due to failure and Party A formally notifies Party B (formal notification includes but not limited to telephone, fax, e-mail, etc.), Party B shall send someone to the site within 24 hours for handling.

设备在发生故障停机后，到甲方正式通知到供方为止的时间 (正式通知包括但不局限于电话、传真、电子邮件等) 后,供方应于24小时之内派人到达现场进行处理。

- Party B has technical support base in Thailand that can provide spare parts service.

乙方在泰国有技术支持基地，可提供便利的备品备件服务。

- Safety requirements安全要求

- The equipment provided by Party B shall comply with the laws, regulations, rules, specifications, standards (including local and industrial standards and guidelines) and requirements related to work safety in China and Thailand, and shall meet the fire safety standards of China and Thailand if fire-fighting is involved, and shall meet the relevant laws and regulations of China and Thailand for special equipment.乙方提供的设备须符合中国、泰国现行的有关安全生产的法律、法规、条例、规范、标准(包括地方、行业等标准和指引)和要求，涉及消防的要满足中国、泰国消防安全标准，涉及特种设备须满足中国、泰国相关法律法规。

- Party B shall ensure the safety performance of the equipment, and the material of the equipment shall be selected based on the application environment, considering the physical and chemical properties of the materials, to avoid the occurrence of safety accidents.乙方应保证设备的安全性能，设备的选材要根据其使用环境，充分考虑其物理、化学特性、避免安全事故的发生。

- For the installation of the equipment, site survey is required. And the installation demands of the equipment shall be fully considered. The supplier is responsible for the implementation of safety measures on the installation site: responsible for construction safety (including equipment safety and personnel safety).设备的安装要考察实地安装环境，充分考虑设备的安装需要，并负责落实安装现场的安全防护措施：对施工安全(包括设备安全和人员安全)负责。

- The installation operators of Party B should be trained on safety knowledge, The operators for special equipment shall obtain operation certificate for special equipment. Within 10 days after the completion of equipment installation and commissioning, the documents must be submitted according to safety specification, such as design documents, product quality certification, installation / operation /maintenance instructions in Chinese (which must include safety precautions, etc.), supervision and inspection certificates.乙方安装作业人员要经过安全知识培训，特种设备作业人员要取得特种设备从业资格，设备安装调试完成后 10 天内必须提交安全技术规范的要求的设计文件，产品质量合格证明，安装及使用维修中文说明(其中必须包括安全注意事项等)，监督检验证明等文件。

- Party B shall provide training to the operation and maintenance personnel on safety knowledge, and submit the training materials and the form with trainees’ signature, and the personnel are not allowed to work until they have passed the training.乙方要对操作、维修人员进行安全知识培训，并提交培训资料和接受培训人员签名表，培训合格后方可上岗。

- Party B shall provide risk warnings for potential safety hazards parts of the equipment, and remind the operating personnel to pay attention to the risks.乙方要对设备可能出现的安全隐患部位做出风险提示，提醒作业人员注意风险。

- Party B shall carry out regular safety inspection to the equipment, and take corrective action to the items that are not in compliance with safety regulations.乙方要对设备定期进行安全检验，并对不符合安全事项进行整改。

- For special equipment, the above items shall meet the relevant standards for safety of special equipment.属于特种设备的，上述各项须满足特种设备安全有关标准。

- If there are safety risks, defects and any violation to the above safety requirements confirmed by the monitoring & management department or Party A existed in the equipment design, manufacturing, installation, inspection, repair and (maintenance) modification, Party B shall unconditionally accept the requirements for corrective action.设备的设计、制造、安装、检验、维修、(保养)改造中涉及监察、管理部门或甲方确认的存在安全隐患、漏洞及上述安全要求的地方，乙方须无条件接受整改要求。

- Protection system for equipment heater设备加热器保护系统：

a)	The heater used for drying and oven must be installed with wind pressure interlock protection and air exhaust interlock protection.用于烘干,烤箱的加热器,必须安装风压互锁保护,抽风互锁保护。

b)	The main circuit of the heater must be installed with leakage protection switch, automatic tripping air switch.加热器的主电路,必须安装漏电保护开关,自动脱扣空气开关。

c)	The control circuit of the heater must be equipped with independent over temperature setting protection and body over temperature protection, the over temperature protection signal directly acts on the automatic breaker to cut off the main power supply.加热器的控制电路,必须安装独立的设定超温保护,机体超温保护,超温保护信号直接作用于自动脱扣器,切断主电源。

d)	The material of heaters shall be selected in line with the application environment conditions, which shall be corrosion resistant, oxidation resistant and ablation resistant accordingly, and comply with "JB/T2379-2016 industrial heating tube standards.加热器的材质选用, 符合应用环境条件, 具有相应的防腐性, 抗氧化性及耐烧蚀, 符合《JB/T2379-2016 工业加热管标准》。

e)	In the heater installation section of the equipment, the materials of the fixing device of the heater and the equipment body should be UL94 V1 or above grade flame retardant materials. The dryer and oven body should be made of stainless steel or iron materials.安装加热器设备机体段,加热器的固定装置,设备机体等材料应具有阻燃性,阻燃等级 UL94 V1 级以上,烘干与烤箱设备机体采用不锈钢或铁质材质。

f)	The heater shall use flame retardant cables, which shall comply with the flame retardant standard for cables: GB/T18380.12-2022.加热器的导线,具有阻燃性,符合电线阻燃标准: GB/T18380.12-2022。

11.	If the above safety requirements are not implemented and executed, Party A has the right to request corrections. The equipment will not be accepted and returned to the supplier if it is refused to take corrective action, and Party B shall be responsible for all losses caused thereby.对没有贯彻及执行上述安全要求的，甲方有权要求进行整改，对拒不进行整改的不予验收和进行退机处理，对由此引起的一切损失由乙方负责。

12.	The personal injury or property loss caused by the safety issues of Party B's equipment shall be borne by Party B.因乙方的设备安全问题造成人员伤亡、财产损失的，其后果由乙方负责。